1. Main PCB Pinout & Connection Table

A. ESP32-C3 SuperMini Pin Allocations

| ESP32-C3 Pin | Connected Net / Component | Function / Signal | Notes |
| --- | --- | --- | --- |
| 5V | 5V Rail | Main Power Entry | USB-C input from power supply; feeds RJ45 Pins 1 & 4 |
| 3.3V | 3.3V Rail | Regulated Power | Powers RC522 RFID module & I2C pull-ups |
| GND | GND Rail | System Ground | Common ground pour across the entire PCB |
| GPIO 0 | RC522 Pin 5 | SPI MISO | Master In, Slave Out |
| GPIO 2 | RC522 Pin 2 | SPI RST | Hardware Reset |
| GPIO 3 | Active Buzzer (+) | Digital Output | Active High Alarm trigger |
| GPIO 4 | Red LED (Anode via R2) | Digital Output | Danger indicator (330 ohm current-limiting resistor to GND) |
| GPIO 5 | Green LED (Anode via R1) | Digital Output | Safe indicator (330 ohm current-limiting resistor to GND) |
| GPIO 6 | RC522 Pin 7 | SPI SCK | Serial Clock |
| GPIO 7 | RC522 Pin 6 | SPI MOSI | Master Out, Slave In |
| GPIO 8 | RJ45 Pin 3 | I2C SDA | Thermal camera data line (2.2k ohm pull-up R3 to 3.3V) |
| GPIO 9 | RJ45 Pin 5 | I2C SCL | Thermal camera clock line (2.2k ohm pull-up R4 to 3.3V) |
| GPIO 10 | RC522 Pin 8 | SPI SS / SDA | Chip Select |
| GPIO 20 | RJ45 Pin 7 | UART RX (Serial1) | Receives radar data from LD2410 TX |
| GPIO 21 | RJ45 Pin 8 | UART TX (Serial1) | Transmits radar configuration to LD2410 RX |

B. MFRC522 RFID Header (8-Pin Header U2)

| Header Pin | Module Pin | Connected Net | Notes |
| --- | --- | --- | --- |
| Pin 1 | 3.3V | 3.3V Rail | Don’t connect to 5V! |
| Pin 2 | RST | ESP32 GPIO 2 | Reset line |
| Pin 3 | GND | GND Rail | Ground |
| Pin 4 | IRQ | NC (Unconnected) | Not used in firmware |
| Pin 5 | MISO | ESP32 GPIO 0 | SPI Data In |
| Pin 6 | MOSI | ESP32 GPIO 7 | SPI Data Out |
| Pin 7 | SCK | ESP32 GPIO 6 | SPI Clock |
| Pin 8 | SDA / SS | ESP32 GPIO 10 | SPI Chip Select |

C. RJ45 LAN Bridge Connector (Cat6 Pinout)

| RJ45 Pin | Main PCB Origin | Remote PCB Destination | Wire Function |
| --- | --- | --- | --- |
| Pin 1 | 5V Rail | Remote 5V Rail | Primary Power Pair |
| Pin 2 | GND Rail | Remote GND Rail | Primary Ground Pair |
| Pin 3 | ESP32 GPIO 8 | AMG8833 SDA | I2C Data Line |
| Pin 4 | 5V Rail | Remote 5V Rail | Secondary Power Pair (Parallel) |
| Pin 5 | ESP32 GPIO 9 | AMG8833 SCL | I2C Clock Line |
| Pin 6 | GND Rail | Remote GND Rail | Secondary Ground Pair (Parallel) |
| Pin 7 | ESP32 GPIO 20 | HLK-LD2410 TX | UART RX |
| Pin 8 | ESP32 GPIO 21 | HLK-LD2410 RX | UART TX |

D. Discrete Passives

| Component | Value / Spec | Connection | Function |
| --- | --- | --- | --- |
| R1 | 330 ohm1/4W | ESP32 GPIO 5 -> Green LED Anode | Green LED current limit |
| R2 | 330 ohm1/4W | ESP32 GPIO 4 -> Red LED Anode | Red LED current limit |
| R3 | 2.2k ohm 1/4W | ESP32 GPIO 8 (SDA) -> 3.3V Rail | I2C SDA Pull-up |
| R4 | 2.2k ohm 1/4W | ESP32 GPIO 9 (SCL) -> 3.3V Rail | I2C SCL Pull-up |
| C1 | 100μF 16V Electrolytic | 5V Rail -> GND Rail | Main power buffer |
| C2 | 100nF Ceramic (104) | 3.3V Rail -> GND Rail | High-frequency noise bypass (RFID) |
| C3 | 10μF 16V Electrolytic | 3.3V Rail -> GND Rail | Low-frequency voltage sag bypass (RFID) |